Variable List and Descriptions

- High-Frequency Sensor Data

Each molding cycle corresponds to one CSV file. Data recording starts at the moment the mold closes and ends at the moment the next cycle begins.

Example CSV filename:

“mold_data_611925088_56694.csv”

Where 611925088 is the cycle timestamp code, corresponding to spcTime in the Injection Molding Machine Status Data and versionTime in the Machine Process Setting Parameters; 56694 is the cycle number, corresponding to Id in both the Injection Molding Machine Status Data and the Machine Process Setting Parameters.

Each file contains 24 columns:

| Name | Description | Remarks |
| --- | --- | --- |
| SampleTime | Sampling Time | From the start of mold closing to the start of mold closing in the next cycle |
| Phase | Action phases of the injection molding machine | Action phases, including: 0–Idle, 1–Mold Closing, 2–Nozzle Forward, 3–Injection, 4–Packing/Holding, 6–Plasticizing, 7–Decompression, 8–Cooling, 9–Nozzle Backward, 10–Mold Opening, 11–Ejector Forward, 12–Ejector Backward, 13–16–Core 1–4 Forward, 17–20–Core 1–4 Backward. |
| Sensor1 | In-cavity Pressure | In-cavity sensor signals |
| Sensor2 |  |  |
| Sensor3 |  |  |
| Sensor5 | In-cavity Temperature |  |
| Sensor6 |  |  |
| Sensor8 | Actual flow rate of the mold temperature controller (water flowmeter) |  |
| SP | Actual screw position |  |
| IJ | Injection pressure at the nozzle |  |
| MouldTemp1 | Temperature of thermal controller 1 | Actual temperature sensor of the mold temperature controller |
| MouldTemp2 | Temperature of thermal controller 2 |  |
| MouldTemp3 | Cooling water temperature |  |
| MouldTemp4 | Heating water temperature |  |
| MouldTemp5 | Return water temperature |  |
| MouldTemp9 | Moving mold temperature | Actual mold core temperature sensor signal |
| MouldTemp10 |  |  |
| MouldTemp11 |  |  |
| MouldTemp12 | Fixed mold temperature |  |
| MouldTemp13 |  |  |
| MouldTemp14 |  |  |
| MouldFlow1 | Total volume per cycle | Actual supplied water flow rate of the mold temperature controller |
| MouldFlow2 | Total cooling water volume per cycle |  |
| MouldFlow3 | Total heating water volume per cycle |  |

- Injection Molding Machine Status Data, spc

| Name | Description | Remarks |
| --- | --- | --- |
| spcTime | Record Time | MES timestamp at the end of the cycle |
| EL_CYC_TIME | Cycle Time |  |
| EL_NZL_MEAN | Average Temperature |  |
| EL_CLAMP_PRESS | Clamping Pressure | System pressure during high-pressure clamping |
| EL_MOP | Mold Opening End Position |  |
| EL_PLST_END_ERR | Plasticizing End Position |  |
| EL_INJ_START_STR | Injection Start Position |  |
| EL_MAX_INJ_PRESS | Maximum Injection Pressure |  |
| EL_IV_END_STR | V/P Switchover Position |  |
| EL_IV_END_TIME | V/P Switchover Time | Does not include the injection start delay time |
| EL_IV_END_PRESS | V/P Switchover Pressure |  |
| EL_VPS_END_STR | Injection End Position |  |
| EL_MIN_INJ_STR | Minimum Shot Position |  |
| op_mode | Operation Mode | 1: Mold setup, 2: Manual, 3: Semi-automatic, 4: Fully automatic |
| EL_PLST_TIME | Plasticizing Time | Does not include decompression time |
| EL_LAST_COOL_TIME | Post-cooling Time | Cooling time after plasticizing ends |
| EL_MC_TIME | Mold Closing Time |  |
| EL_MO_TIME | Mold Opening Time |  |
| EL_EF_TIME | Ejector Forward Time |  |
| EL_EB_TIME | Ejector Backward Time |  |
| EL_IDLE_TIME | Idle Time |  |
| EL_CORE_IN_TIME_<Nr> | Core Forward Time | Nr ranges from 0 to 4 |
| EL_CORE_OUT_TIME_<Nr> | Core Backward Time | Nr ranges from 0 to 4 |
| EL_IV_TIME | Filling Time |  |
| EL_PP_TIME | Packing/Holding Time |  |
| EL_SB1_TIME | Pre-decompression Time |  |
| EL_SB2_TIME | Post-decompression Time |  |
| EL_CF_TIME | Nozzle Forward Time |  |
| EL_CB_TIME | Nozzle Backward Time |  |
| EL_BARREL_MEAN_<Nr> | Barrel Temperature | Nr ranges from 1 to 7. Sampled once per second, averaged over the entire cycle |
| EL_HTRN_TI_<Nr> | Hot Runner Temperature | Nr ranges from 0 to 7. Value at the end of the cycle |
| EL_PLST_BP_<Nr> | Plasticizing Back Pressure | Nr ranges from 0 to 4 Sampled every 0.01 s; average over the last 0.1 s of each stage, representing a steady-state value |
| EL_PP_STG_POS_<Nr> | Packing End Position | Nr ranges from 0 to 4; position at the end of packing/holding for each segment |
| EL_IV_STG_TIME_<Nr> | Filling Time | Nr ranges from 0 to 4 |
| EL_EF_END_POS | Ejector Forward End Position |  |
| EL_CF_END_PRESS | Nozzle Forward End Pressure |  |
| Id | Cycle Id | Number of cycles produced by the machine |
| remark | Remarks |  |
| ext_mold_temp_flt_<Nr> | Mold Temperature | Nr ranges from 0 to 4. Sampled once per second, averaged over the entire cycle, including: Thermal Controller 1, Thermal Controller 2, Cooling Water, Heating Water, and Return Water (°C). |
| ext_mold_flow_flt_<Nr> | Mold Water Flow Rate | Nr ranges from 0 to 3. Value at the end of the cycle, including: Instantaneous flow rate, Total cooling water volume per cycle, Total volume per cycle, and Total heating water volume per cycle (L). |
| ext_mold_time_flt_<Nr> | Mold Water Flow Time | Nr ranges from 0 to 4 (s), including: Total time per cycle, Cooling water time per cycle, Heating water time per cycle, Switching time from heating to cooling water, and switching time from cooling to heating water; value at the end of the cycle. |
| rob_enbl | Robot Usage |  |

- Machine Process Setting Parameters, dataset

| Name | Description | Remarks |
| --- | --- | --- |
| techType | Process Type | 1. currentValue is the current value. When inserting into dbspc, if productType is machine setup adjustment, then one row will be inserted; 2. standardValue is the standard value, and standardDeviation is the standard deviation, which are generated when submitting in iMark; 3. specifiedTimeValue refers to values at specific times, 8:00 AM and 8:00 PM 4. realTimeValue is the real-time valu |
| versionTime | Record Time | This value is the same as spcTime in the spc database |
| remark | Remarks |  |
| adjust_reasons | Reason for Machine Setup Adjustment | Format: “scrap type ID,”. The meaning of the scrap type ID can be found in the DBFactoryBadProductType table, which is defined in the factory information settings of iMark |
| Id | Cycle Id (Number of cycles produced by the machine)） | This value is the same as Id in the spc database |
| mc_press_<Nr> | Mold Closing Pressure | Nr refers to the sub-stage: 0 for Mold Closing 1, 1 for Mold Closing 2, 2 for Mold Protection, 3 for High Pressure. High pressure has no position |
| mc_speed_<Nr> | Mold Closing Speed |  |
| mc_pos_<Nr> | Mold Closing End Position |  |
| mo_press_<Nr> | Mold Opening Pressure | Nr refers to the sub-stage: 2 for Unlocking, 3 for High Speed, 4 for Medium Speed, 5 for Low Speed |
| mo_speed_<Nr> | Mold Opening Speed |  |
| mo_pos_<Nr> | Mold Opening End Position |  |
| ef_press_<Nr> | Ejector Forward Pressure | Nr refers to the sub-stage: 0 for Segment 1, 1 for Segment 2, 2 for Segment 3 |
| ef_speed_<Nr> | Ejector Forward Speed |  |
| ef_time_<Nr> | Ejector Forward Time |  |
| ef_pos_<Nr> | Ejector Forward End Position |  |
| ef_points | Number of Ejector Forward Segments |  |
| ef_times | Number of Ejections |  |
| eje_oprt_mode | Ejector Pin Mode | 0 means "Not used"; 1 means "Dwell"; 2 means "Vibration"; 3 means "Multiple times"; |
| phs_start_delay_10 | Ejector Forward Start Delay |  |
| eb_press_<Nr> | Ejector Backward Pressure | Nr refers to the sub-stage: 0 for Segment 1, 1 for Segment 2, 2 for Segment 3 |
| eb_speed_<Nr> | Ejector Backward Speed |  |
| eb_time_<Nr> | Ejector Backward Time |  |
| eb_pos_<Nr> | Ejector Backward End Position |  |
| eb_points | Number of Ejector Backward Segments |  |
| phs_start_delay_11 | Ejector Backward Start Delay |  |
| core_in_press_<Nr> | Core In Pressure | Nr refers to the termination index: 0–3 correspond to Core 1 to Core 4 Remarks: core-related settings such as mode, action position, priority, etc., are not included mode Control mode: 0 indicates time, 1 indicates limit switch, 2 indicates gear count |
| core_in_speed_<Nr> | Core In Speed |  |
| core_in_mode_<Nr> | Core In Control Mode |  |
| core_in_time_<Nr> | Core In Time |  |
| core_out_press_<Nr> | Core Out Pressure |  |
| core_out_speed_<Nr> | Core Out Speed |  |
| core_out_mode_<Nr> | Core Out Control Mode |  |
| core_out_time_<Nr> | Core Out Time |  |
| cf_press_<Nr> | Nozzle Forward Pressure | Nr refers to the sub-stage: 0 for high speed, 1 for low speed |
| cf_speed_<Nr> | Nozzle Forward Speed |  |
| cf_pos_<Nr> | Nozzle Forward End Position |  |
| cf_ls_delay | Nozzle Forward Time |  |
| cb_press | Nozzle Backward Pressure |  |
| cb_speed | Nozzle Backward Speed |  |
| cb_time | Nozzle Backward Time |  |
| cb_pos | Nozzle Backward End Position |  |
| pack_press_<Nr> | Packing Pressure | Nr refers to the sub-stage: 0–4 correspond to Packing/Holding Segment 1 to Segment 5 |
| pack_speed_<Nr> | Packing Speed |  |
| pack_time_<Nr> | Packing Time |  |
| vps_time | Packing Switchover Time |  |
| vps_str | Packing Switchover Position |  |
| vps_prss | Packing Switchover Pressure |  |
| vps_by_time_enbl | Packing/Holding Switchover Enabled by Time |  |
| vps_by_str_enbl | Packing/Holding Switchover Enabled by Position |  |
| vps_by_prss_enbl | Packing/Holding Switchover Enabled by Pressure |  |
| inj_press_<Nr> | Injection Pressure | Nr refers to the sub-stage: 0–4 correspond to Injection Segment 1 to Segment 5; Nr = 4 has no end position or time |
| inj_speed_<Nr> | Injection Speed |  |
| inj_pos_<Nr> | Injection End Position |  |
| inj_time_<Nr> | Injection Time |  |
| inj_points | Number of Injection Segments |  |
| pack_points | Number of Packing/Holding Segments |  |
| inj_ctr_by_time_enbl | Inter-segment Switchover Mode for Injection | 0 indicates position, 1 indicates time |
| inj_sn_func_<Nr> | Injection Auxiliary Valve Control Mode | Nr indicates which auxiliary valve (0–14): injection boost, injection fast, hydraulic nozzle, packing decompression, gas-assisted injection, hot runner 1 to 10 Mode definition: 0 not used, 1 time open–time close; 2 time open–position close; 3 position open–time close; 4 position open–position close; 5 position open–timed close |
| inj_on_time_<Nr> | Injection Auxiliary Valve Open Time (Injection) |  |
| inj_off_time_<Nr> | Injection Auxiliary Valve Close Time (Injection) |  |
| inj_on_pos_<Nr> | Injection Auxiliary Valve Open Position (Injection) |  |
| inj_off_pos_<Nr> | Injection Auxiliary Valve Close Position (Injection) |  |
| pack_on_time_<Nr> | Injection Auxiliary Valve Open Time (Packing) |  |
| pack_off_time_<Nr> | Injection Auxiliary Valve Close Time (Packing) |  |
| sb<Nr>_mode | Decompression Mode | Nr indicates which decompression: pre-decompression 1, post-decompression 2 |
| sb<Nr>_press | Decompression Pressure |  |
| sb<Nr>_speed | Decompression Speed |  |
| sb<Nr>_str | Decompression Distance |  |
| sb<Nr>_time | Decompression Time |  |
| plst_press_<Nr> | Plasticizing Pressure | Nr refers to the sub-stage: 0–4 correspond to Plasticizing Segment 1 to Segment 5 |
| plst_speed_<Nr> | Plasticizing Screw Speed |  |
| plst_pos_<Nr> | Plasticizing End Position |  |
| plst_bp_<Nr> | Plasticizing Back Pressure |  |
| plst_points | Number of Plasticizing Segments |  |
| phs_start_delay_0 | Idle Time |  |
| cooling_mode | Cooling Mode | 0 indicates after injection, 1 indicates after plasticizing |
| cooling_time | Cooling Time |  |
| fb_sp_<Nr> | Barrel Temperature | Nr indicates which segment: 0–7 (nozzle, segment 1 to segment 7) |
| mdt_sp_<Nr> | Mold Temperature | 0–7 correspond to: 0–4 are Thermal Controller 1, Thermal Controller 2, cooling water, heating water, return water. |
| mdt_time_sp_<Nr> | Mold Temperature Running Time | 0–7 correspond to: 0 indicates interlock heating waiting time, 1 indicates interlock heating time, 2 indicates draining time, 3 indicates interlock cooling waiting time, 4 indicates interlock cooling time |
| hotruner_sp_<Nr> | Hot Runner Temperature | 0–7 |